STUDY THE DATASHEET OF 16X2 LCD

Youtube Video Link

Liquid Crystal Displays (LCDs)

1. Introduction:

- LCDs are flat-panel displays widely used in electronic devices for visual output.

- They utilize the properties of liquid crystals to control the passage of light, enabling the display of text, numbers, and graphics.

2. Functions & Features:

- Display text, numbers, and graphical patterns.

- Low power consumption.

- Lightweight and compact.

- Can operate in a wide range of temperatures.

- Suitable for both indoor and outdoor applications.

- Can be interfaced with microcontrollers and other digital devices for visual output.

3. Mechanism:

- LCDs consist of liquid crystal cells sandwiched between two polarizing sheets.

- The liquid crystal cells change orientation when subjected to an electric field, controlling the passage of light through the display.

- By selectively applying voltage to specific segments of the display, characters, numbers, or graphics can be formed.

4. Pins Description:

- LCD modules typically have multiple pins for power, data, and control signals.

- Common pins include VCC (power supply), GND (ground), RS (Register Select), RW (Read/Write), EN (Enable), and data pins for sending commands and data to the display.

5. Electrical Characteristics:

- Operating Voltage: Typically 5V or 3.3V.

- Operating Current: Depends on the display size and backlight configuration.

- Interface: Parallel or serial communication with microcontrollers.

- Backlight: Some LCDs feature built-in LED backlighting for enhanced visibility in low-light conditions.

6. Quality Specifications:

  - Resolution: Defines the number of pixels or characters that can be displayed.

  - Contrast Ratio: The ratio of the luminance of the brightest color to that of the darkest color the system is capable of producing.

  - Response Time: The time taken for a pixel to transition from one state to another.

  - Viewing Angle: The maximum angle at which the display can be viewed with acceptable visual performance.

  - MTBF (Mean Time Between Failures): A measure of the expected reliability of the display module.

7. Other Important Considerations:

  - Compatibility: Ensure compatibility between the LCD module and the interfacing device (e.g., microcontroller).

  - Programming: Write code to control the LCD and send commands/data for displaying desired content.

  - Environmental Conditions: Consider operating temperature, humidity, and other environmental factors when selecting an LCD for a specific application.

  - Mounting and Enclosure: Provide appropriate mounting and enclosure to protect the LCD from physical damage and environmental hazards.

In summary, LCD displays are versatile visual output devices suitable for a wide range of applications. Understanding their functions, features, mechanism, pin descriptions, electrical characteristics, quality specifications, and other important considerations is essential for effectively integrating them into electronic projects and systems.

Guide to Connecting and Using an LCD with an 8051 Microcontroller

Sure! Here’s a step-by-step explanation of how to connect an LCD to a microcontroller (such as the 8051) and print characters on it:

### 1. Understand the LCD Pins

A typical 16x2 LCD has 16 pins, but the most important ones are:

- VSS: Ground

- VDD: Power supply (typically +5V)

- VO: Contrast adjustment

- RS: Register Select (0 for command, 1 for data)

- RW: Read/Write (0 for write, 1 for read)

- E: Enable

- D0-D7: Data pins

- A and K: Backlight (Anode and Cathode)

### 2. Connect Power and Contrast

- Connect VSS to ground.

- Connect VDD to +5V.

- Connect VO to a potentiometer (middle pin) to adjust the display contrast.

### 3. Connect Control Pins

- Connect the RS, RW, and E pins to three GPIO pins on the microcontroller.

  - For example, RS to P2.0, RW to P2.1, and E to P2.2 on an 8051 microcontroller.

### 4. Connect Data Pins

- For 4-bit mode (commonly used to save GPIO pins):

  - Connect only the higher four data pins (D4-D7) to four GPIO pins on the microcontroller.

  - For example, D4 to P2.4, D5 to P2.5, D6 to P2.6, and D7 to P2.7.

- For 8-bit mode:

  - Connect all eight data pins (D0-D7) to eight GPIO pins on the microcontroller.

### 5. Initialize the LCD

Initialization involves sending a series of commands to the LCD to set it up. Here’s the sequence:

- Power on delay: Wait for the LCD to become ready (typically 20ms).

- Function set: Send the command to set 4-bit mode, number of lines, and character font.

- Display control: Turn on the display and set cursor and blinking options.

- Entry mode set: Set the cursor move direction (increment) and display shift.

- Clear display: Clear the display and move the cursor to the home position.

### 6. Send Commands and Data

- Command Mode: Set RS to 0, RW to 0, place the command on the data pins, and pulse the E pin.

- Data Mode: Set RS to 1, RW to 0, place the character data on the data pins, and pulse the E pin.

### 7. Print Characters

- To print characters, send the character data in ASCII format to the LCD in data mode.

### Example Steps for Initialization and Printing:

- Initialization Sequence (in 4-bit mode):

  - Send 0x02 to initialize the LCD in 4-bit mode.

  - Send 0x28 for 2-line display, 5x8 font.

  - Send 0x0C to turn on the display and turn off the cursor.

  - Send 0x06 to set entry mode (increment cursor, no display shift).

  - Send 0x01 to clear the display.

- Sending Data to Display 'HELLO':

  - Set RS to 1 (data mode).

  - Send ASCII values for 'H', 'E', 'L', 'L', 'O' sequentially.

### Summary of Connections:

- Power:

  - VSS to GND

  - VDD to +5V

  - VO to potentiometer (contrast control)

- Control Pins:

  - RS to P2.0

  - RW to P2.1

  - E to P2.2

- Data Pins (for 4-bit mode):

  - D4 to P2.4

  - D5 to P2.5

  - D6 to P2.6

  - D7 to P2.7

### Summary of Commands:

- Initialization:

  - 0x02: 4-bit mode initialization

  - 0x28: 4-bit mode, 2 lines, 5x8 font

  - 0x0C: Display ON, cursor OFF

  - 0x06: Entry mode set

  - 0x01: Clear display

- Printing 'HELLO':

  - ASCII values: 'H' (0x48), 'E' (0x45), 'L' (0x4C), 'L' (0x4C), 'O' (0x4F)

By following these steps and understanding the sequence of commands, you can connect an LCD to an 8051 microcontroller and display characters on it.

LCD 1602h to MCU:

As in the below diagram, it is shown that LCD data/commands pins are connected to the MCU Port 0.